Fall Protection Quiz

1) If there is any chance of free-falling, do not use a…

- fall arrest system.

- full body harness.

- lanyard.

- chest harness.

2) "Rolling out" is a condition when…

- one goes out for a pass.

- we use the red carpet.

- the keeper on a snap-hook is forced open.

- we wake up in the morning.

3) Means of rescue after a fall could be…

- manlifts.

- ladders.

- All of these are correct.

- Emergency Response Team.

4) Attach full body harnesses to lanyard at the center of the chest.

- True

- False

5) The anchor point should be ______ your chest.

- lower than

- level with or higher than

- adjacent to

6) Ropes, straps, and webbing must be constructed of synthetic fiber.

- True

- False

7) Harnesses are to be made from…

- wire rope.

- 3/4 in. hemp.

- cable.

- synthetic fiber.

8) Personal fall arrest equipment must be inspected…

- before each use.

- annually.

- monthly.

- semi-annually.

9) Deceleration distance is usually around 4 feet.

- True

- False

10) Fall protection is required when working on an unprotected surface over____ from the next level.

- 5 ft.

- 4 ft.

- 8 ft.

- 6 ft.

11) Tie-off _________ you get into a position where you could fall.

- before

- at the time

- after

- while

12) Anchor points used for fall arrest equipment should be capable of supporting at least 500 pounds.

- True

- False

13) When a fall occurs, the person is in…

- terminal velocity.

- deceleration.

- free fall.

- a bind.

KEY

- d

- c

- c

- b

- b

- a

- d

- a

- a

- b

- a

- b

- c